|  | SRI RAMAKRISHNA INSTITUTE OF TECHNOLOGY (An Autonomous Institution) (Approved by AICTE, New Delhi & Permanently Affiliated to Anna University, Chennai) Accredited by NAAC with ‘A’ Grade Pachapalayam, Perur Chettipalayam, Coimbatore-641010 |  |
| --- | --- | --- |

20EEP34/ Introduction to Raspberry Pi

PROJECT – III

Car Safety Control and Airbag Notification Alert System

using Raspberry Pi 4

Submitted By:

Akash A – 71382303001

Amuthan A – 71382303002

Gowri B – 71382303006

Hariharasudhan M - 71382303007

Sharmili E – 71382303018

Tharun B S – 71382303020

Guided By:

Mr. E. Ramkumar

AP (Sl. Gr)/EEE

Assessment

| Description | Marks | Marks Obtained |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Problem Statement | 20 |  |  |  |  |  |  |
| Circuit & Discussion | 30 |  |  |  |  |  |  |
| Students Name |  | 2303001 | 2303002 | 2303006 | 2303007 | 2303018 | 2303020 |
| Presentation | 30 |  |  |  |  |  |  |
| Q/A | 20 |  |  |  |  |  |  |
| Total | 100 |  |  |  |  |  |  |

SIGNATURE

Introduction:

The Car Safety Control and Airbag Notification Alert System leverages the Raspberry Pi as a central processing unit to monitor vehicle conditions and driver behavior. It addresses critical road safety concerns by combining accident detection, automatic airbag inflation, and emergency notification. With road accidents causing over 1.3 million deaths globally each year, such systems are vital for reducing fatalities and improving emergency response times. The system uses a vibration sensor to detect impacts, triggers an electric air pump to inflate airbags, and sends SMS alerts through a GSM module to designated emergency contacts. Additionally, it includes driver drowsiness detection using a camera and OpenCV for facial analysis, enhancing overall safety.

Problem Statement:

Road accidents remain a leading cause of death and injury worldwide, often exacerbated by delayed emergency response and lack of real-time accident detection. Many vehicles lack integrated systems that can automatically detect collisions, deploy safety mechanisms like airbags, and notify emergency services with precise location data. Human factors such as driver fatigue and intoxication further increase accident risks. Existing systems may not provide immediate alerts or confirm airbag deployment, leading to potential failures in critical situations. This project aims to solve these issues by creating an automated, reliable, and responsive safety system that detects accidents, inflates airbags, and sends instant notifications with GPS coordinates if integrated, ensuring faster medical and police intervention.

Components Required:

- Raspberry Pi 4 Model B (4GB)​

- Raspberry Pi Camera Module V2​

- GSM Module (SIM800L)​

- Vibration Sensor (SW-420 module)​

- 12V DC mini air pump​

- Inflatable air wedge/airbag​

- 1-Channel 5V relay module (AC switch)​

- Limit switch (lever/roller type)​

- 16x2 LCD with I2C interface​

- Regulatory circuitry (5V regulation for Pi)​

- PCB board, resistors, capacitors, transistors, and cables

Working Principle:

The system operates in three main phases: drowsiness detection, accident detection, and emergency response. First, the Pi camera monitors the driver’s face using OpenCV to detect signs of drowsiness such as eye closure or head droop, triggering an audible alert and uploading a snapshot to an IoT page. Second, upon impact, the SW-420 vibration sensor detects sudden jerks and signals the Raspberry Pi to activate the 12V air pump, inflating the airbag to protect passengers. A limit switch confirms full inflation; if not achieved within a set time, a relay cuts power and triggers a secondary alert to police, indicating potential system failure. Simultaneously, the GSM module sends SMS alerts to pre-registered numbers (hospital and police), including accident details.

Hardware and Software Used:

The hardware includes the Raspberry Pi 4 Model B as the central controller, interfaced with a Pi Camera Module V2 for drowsiness detection, a SIM800L GSM module for SMS alerts, an SW-420 vibration sensor for impact detection, a 12V DC air pump for airbag inflation, and a 16x2 LCD with I2C for status display. The limit switch and relay module ensure fail-safe operation. Software-wise, the system runs on Raspbian OS, utilizing Python for programming logic, OpenCV for image processing, and dlib for facial landmark detection. The Node-RED tool may also be used for visual programming and system integration.

Libraries Used:

The system relies on several Python libraries: OpenCV for real-time video capture and facial analysis, dlib for detecting facial landmarks such as eyes and mouth, and picamera for interfacing with the Raspberry Pi camera. The RPi.GPIO library controls GPIO pins for sensor and actuator interfacing, while pyserial enables communication with the GSM module via UART. Additional libraries include time and datetime for timing operations, and smbus for I2C communication with the LCD display.

Applications and Use Cases:

This system is applicable in automotive safety prototypes, driver monitoring systems, and intelligent transportation systems. It can be used in educational institutions for teaching embedded systems and IoT applications. Fleet management companies can deploy it to monitor driver behavior and ensure safety compliance. Insurance providers may use it for risk assessment and claims verification. Public transportation systems can integrate it to enhance passenger safety, while parents can use it to monitor teenage drivers. The system also serves as a foundation for advanced driver assistance systems (ADAS) and autonomous vehicle safety protocols.

Advantages & Limitations:

Advantages:

The system offers real-time accident detection and automatic emergency response, significantly reducing response time and increasing survival chances. It provides dual safety features—drowsiness detection and impact response—enhancing overall driver and passenger protection. The use of low-cost, readily available components makes it affordable and scalable. Being open-source and customizable, it allows for integration with GPS for location tracking and expansion with additional sensors like alcohol detectors. The fail-safe mechanism with limit switch feedback ensures reliability even in partial system failures.

Limitations:

The prototype does not include GPS for real-time location tracking, limiting the precision of emergency alerts. The airbag inflation relies on a 12V pump, which may be slower than pyrotechnic airbag systems used in commercial vehicles. The system requires a stable power supply and may fail during electrical disruptions. Environmental factors like extreme temperatures or humidity could affect sensor accuracy. Additionally, the drowsiness detection algorithm may produce false positives due to lighting conditions or driver posture. The current design is a standalone prototype and not integrated with actual vehicle systems like the ECU.

Future Scope:

Future enhancements include integrating GPS for accurate location sharing during accidents, enabling emergency services to reach the site faster. The system can be upgraded with alcohol and seatbelt sensors to prevent drunk driving and ensure compliance. Machine learning models can improve drowsiness detection accuracy by analyzing multiple behavioral cues. Integration with vehicle telematics and cloud platforms can enable remote monitoring and data analytics. The system could also support voice calls or mobile app notifications alongside SMS. Long-term, it can evolve into a comprehensive ADAS with collision avoidance, lane departure warning, and adaptive cruise control features.

Conclusion:

The Car Safety Control and Airbag Notification Alert System using Raspberry Pi presents a cost-effective, intelligent solution for enhancing road safety. By combining sensor-based accident detection, automatic airbag deployment, and GSM-based emergency alerts, it addresses critical gaps in current vehicle safety systems. The integration of drowsiness monitoring further strengthens its preventive capabilities. While the current prototype has limitations, its modular design allows for future scalability and integration with advanced technologies. This system demonstrates the potential of embedded systems and IoT in saving lives and reducing the impact of road accidents through timely intervention.

Appendix:

Python Code with Explanation:

import cv2

import numpy as np

import time

import smbus

import serial

from picamera import PiCamera

from time import sleep

from RPi import GPIO

# GPIO Setup

GPIO.setmode(GPIO.BCM)

VIBRATION_SENSOR_PIN = 17

RELAY_PIN = 18

LIMIT_SWITCH_PIN = 27

GPIO.setup(VIBRATION_SENSOR_PIN, GPIO.IN)

GPIO.setup(RELAY_PIN, GPIO.OUT)

GPIO.setup(LIMIT_SWITCH_PIN, GPIO.IN)

# GSM Setup

ser = serial.Serial('/dev/ttyS0', 9600, timeout=1)

ser.write(b'AT\r')

time.sleep(1)

# Camera Setup

camera = PiCamera()

camera.resolution = (640, 480)

camera.framerate = 32

# Load Haar Cascade for eye detection

eye_cascade = cv2.CascadeClassifier('/home/pi/opencv/data/haarcascades/haarcascade_eye.xml')

def send_sms(phone_number, message):

ser.write(b'AT+CMGF=1\r')

time.sleep(1)

ser.write(f'AT+CMGS="{phone_number}"\r'.encode())

time.sleep(1)

ser.write(message.encode() + b'\x1A')

time.sleep(1)

def detect_drowsiness():

ret, frame = camera.read()

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

eyes = eye_cascade.detectMultiScale(gray, 1.3, 5)

if len(eyes) == 0:

# Eyes not detected - possible drowsiness

print("Drowsiness detected!")

# Trigger alert

send_sms("HOSPITAL_NUMBER", "Driver drowsiness detected!")

camera.capture('/home/pi/drowsy_driver.jpg')

# Upload image to IoT platform (code not shown)

def main():

while True:

vibration = GPIO.input(VIBRATION_SENSOR_PIN)

if vibration == 0:  # Vibration detected

print("Accident detected!")

# Activate air pump via relay

GPIO.output(RELAY_PIN, GPIO.HIGH)

time.sleep(1)

# Check if airbag inflated within 5 seconds

start_time = time.time()

while (time.time() - start_time) < 5:

if GPIO.input(LIMIT_SWITCH_PIN) == 1:

print("Airbag inflated successfully")

send_sms("HOSPITAL_NUMBER", "Accident detected! Airbag deployed. Location: [GPS DATA]")

break

else:

# Airbag not inflated

print("Airbag failed to inflate")

send_sms("POLICE_NUMBER", "Accident detected! Airbag failed. Immediate assistance required.")

# Reset relay

GPIO.output(RELAY_PIN, GPIO.LOW)

# Check for drowsiness every 30 seconds

time.sleep(30)

detect_drowsiness()

if __name__ == "__main__":

try:

main()

except KeyboardInterrupt:

GPIO.cleanup()