|  | SRI RAMAKRISHNA INSTITUTE OF TECHNOLOGY (An Autonomous Institution) (Approved by AICTE, New Delhi & Permanently Affiliated to Anna University, Chennai) Accredited by NAAC with ‘A’ Grade Pachapalayam, Perur Chettipalayam, Coimbatore-641010 |  |
| --- | --- | --- |

20EEP34/ Introduction to Raspberry Pi

PROJECT – IV

Fire Detection System and Extinguishing Robot using Raspberry Pi 4

Submitted By:

Akash A – 71382303001

Amuthan A – 71382303002

Gowri B – 71382303006

Hariharasudhan M - 71382303007

Sharmili E – 71382303018

Tharun B S – 71382303020

Guided By:

Mr. E. Ramkumar

AP (Sl. Gr)/EEE

Assessment

| Description | Marks | Marks Obtained |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Problem Statement | 20 |  |  |  |  |  |  |
| Circuit & Discussion | 30 |  |  |  |  |  |  |
| Students Name |  | 2303001 | 2303002 | 2303006 | 2303007 | 2303018 | 2303020 |
| Presentation | 30 |  |  |  |  |  |  |
| Q/A | 20 |  |  |  |  |  |  |
| Total | 100 |  |  |  |  |  |  |

SIGNATURE

Introduction:

Fire accidents are among the most devastating disasters, responsible for significant loss of life and property every year. Deploying human firefighters in hazardous environments like chemical factories or nuclear facilities poses extreme risk. A fire-detection and extinguishing robot powered by Raspberry Pi offers an intelligent and autonomous solution to deal with such situations. The system identifies fire using sensors or live video feed, navigates toward it, and sprays water or extinguishing fluid to suppress the flames. Additionally, it can send real-time updates or images to operators through IoT integration for remote monitoring and control.

Problem Statement:

Traditional firefighting methods depend heavily on human intervention, putting firefighters in life-threatening conditions. Conventional smoke and heat sensors provide delayed response times, often detecting fires only after they have spread significantly. Moreover, many automated systems lack localization and extinguishing functionality. Hence, there is a need for a compact, intelligent, fully automated fire detection and control system that can accurately identify flames, navigate towards them, and extinguish the fire efficiently using Raspberry Pi as a central controller.

Components Required:

- Raspberry Pi 4/3 Model B for system control and processing​

- Flame sensors (three or more) placed at different angles for directional flame detection​

- Temperature sensor (DHT11 or MLX90614) for environmental heat measurement​

- Motor driver module (L293D) to control DC motor movements​

- DC motors for robot mobility​

- Servo motor for nozzle rotation and spray control​

- Pump motor for water ejection​

- Pi Camera Module or USB camera for image-based fire detection​

- Relays, buzzer, and LED indicators for system feedback​

- Power supply (12V rechargeable battery)​

- Optional IoT module (Wi-Fi or GSM) for web-based status monitoring

Working Principle:

The robot operates autonomously using flame detection sensors or image processing to detect fires in its vicinity. Flame sensors detect infrared radiation emitted by fire; if fire is sensed, the sensor’s analog signal indicates direction. The Raspberry Pi calculates the position and commands the motor driver (L293D) to orient the robot toward the fire’s source. Once aligned, it activates the pump motor to spray water until the flame intensity drops below the detection threshold. If equipped with a Pi camera, the system uses OpenCV to analyze frames in real time, isolating fire-colored regions (red, orange, yellow) in the HSV color spectrum. The robot continuously monitors for new ignition points, allowing multiple fire suppression cycles.

Hardware and Software Used:

Hardware: Raspberry Pi (controller), L293D motor driver, Pi camera, flame sensors, DHT11 sensor, DC motors, 12V water pump, and relay modules.​
Software: Raspberry Pi OS (Raspbian), Python IDE (Thonny), OpenCV for image processing, and Flask or HTML-based web interface for IoT control. The motor control and sensor integration utilize Python’s GPIO library. For machine-learning fire detection, TensorFlow or YOLOv2 models can be implemented on Pi for advanced image recognition.

Libraries Used:

- OpenCV (cv2): For color-based fire detection and contour tracking​

- RPi.GPIO: To handle motor and sensor interfacing​

- dlib / TensorFlow: For deep-learning fire recognition (optional)​

- NumPy: For matrix operations during frame processing​

- time and serial: For sensor timing and communication with sub-modules​

- requests / Flask: For IoT communication via web interface

Applications and Use Cases:

- Industrial fire prevention in chemical factories, refineries, and warehouses​

- Smart buildings, schools, and malls for safety automation​

- Fire-prone military areas or power plants with restricted human entry​

- IoT-based home automation systems for early fire detection​

- Research and educational projects in robotics and automation fields

Advantages & Limitations:

Advantages:

- Fully autonomous fire detection and suppression without human intervention​

- Real-time flame direction identification and precision water targeting​

- Reduces firefighter risk and response time​

- Integrates IoT for remote monitoring and control​

- Works in multiple environments using both sensor-based and image-based detection​

- Cost-effective and easily replicated with off-the-shelf components.

Limitations:

- Water-based suppression unsuitable for electrical or chemical fires​

- Limited battery capacity restricts operation time​

- Camera-based detection may give false positives under similar color lighting​

- Requires line-of-sight detection—hidden fires may go unnoticed​

- Processing delays using Raspberry Pi for image recognition in high-resolution video​

Future Scope:

- Integration with GPS for location reporting and fire mapping​

- Use of AI models (YOLOv5, CNN) to enhance flame recognition accuracy​

- Implementation of different extinguishing agents (CO₂, foam-based)​

- Deployment of swarm robots coordinating to handle large-scale fires​

- Use of cloud services for data logging and predictive analytics​

- Incorporation of industrial-grade IR/UV flame sensors for higher range​

Conclusion:

The Fire Detection System and Extinguishing Robot using Raspberry Pi represents a significant step forward in intelligent safety and automation. It combines flame sensing, autonomous navigation, and extinguishing capability to prevent fire hazards efficiently. The Raspberry Pi serves as a low-cost yet powerful platform to integrate sensors, motors, and image-based AI models for autonomous hazard control. The project demonstrates potential for industrial and residential adoption, improving safety standards while minimizing human exposure to danger.

Appendix:

Python Code with Explanation:

import cv2

import numpy as np

import RPi.GPIO as GPIO

import time

# Pin configuration

FLAME_LEFT = 17

FLAME_CENTER = 27

FLAME_RIGHT = 22

PUMP_PIN = 23

MOTOR_LEFT = 24

MOTOR_RIGHT = 25

BUZZER = 12

# Setup

GPIO.setmode(GPIO.BCM)

for pin in [FLAME_LEFT, FLAME_CENTER, FLAME_RIGHT, PUMP_PIN, MOTOR_LEFT, MOTOR_RIGHT, BUZZER]:

GPIO.setup(pin, GPIO.OUT if pin in [PUMP_PIN, MOTOR_LEFT, MOTOR_RIGHT, BUZZER] else GPIO.IN)

# Camera initialization

cap = cv2.VideoCapture(0)

fire_detected = False

def move_left():

GPIO.output(MOTOR_LEFT, True)

GPIO.output(MOTOR_RIGHT, False)

def move_right():

GPIO.output(MOTOR_LEFT, False)

GPIO.output(MOTOR_RIGHT, True)

def stop():

GPIO.output(MOTOR_LEFT, False)

GPIO.output(MOTOR_RIGHT, False)

def extinguish_fire():

GPIO.output(PUMP_PIN, True)

GPIO.output(BUZZER, True)

time.sleep(5)

GPIO.output(PUMP_PIN, False)

GPIO.output(BUZZER, False)

while True:

ret, frame = cap.read()

hsv = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

lower = np.array([18, 50, 50])

upper = np.array([35, 255, 255])

mask = cv2.inRange(hsv, lower, upper)

contours, _ = cv2.findContours(mask, cv2.RETR_TREE, cv2.CHAIN_APPROX_SIMPLE)

if len(contours) > 0:

fire_detected = True

cnt = max(contours, key=cv2.contourArea)

(x, y, w, h) = cv2.boundingRect(cnt)

cv2.rectangle(frame, (x, y), (x + w, y + h), (0, 0, 255), 2)

if x < 200:

move_left()

elif x > 400:

move_right()

else:

stop()

extinguish_fire()

else:

fire_detected = False

stop()

cv2.imshow("Fire Detection", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

GPIO.cleanup()